Duet Module Interface Specification

for the

Alero-AEC-8 Audio Mixer System

# TABLE OF CONTENTS

Overview	4

Implementation	5

Channels	9

Levels	10

Command Control	11

Command Feedback	12

Programming Notes	13

Protocol Notes	13

# Revision History

| ###### Date | Initials | Version | Comments |
| --- | --- | --- | --- |
| 05-01-2015 | JRS | V1.0.0 | Initial release |
| 06-08-2015 | JRS | V1.0.1 | Added RAMP Step properties |

Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a Alero family of devices. The Alero supports IP protocol

This module was written using Café Duet firmware version 3.60.453, NetLinx Studio version 3.3.1.525, Café Duet application platform and runtime version 1.15.230, and Café Duet application version 1.8.85.

In the case of IP control, the Comm. module must be informed of the IP address and Port number the device uses for communication. This is set in the module using the PROPERTY command. This may be configured manually via a telnet session, or can be automated from within the UI module. In order to set the IP address you must use the ‘PROPERTY-IP_Address‘ command. The receiver’s communication port defaults to port 2511. Once the IP_Address property are assigned the ‘REINIT’ command is used to set the new parameters within the Comm. module. When these commands are used in the appropriate order, the Comm. module will attempt to connect to the specified IP address periodically until a connection is made or a new IP address and Port number are submitted using the ‘PROPERTY-‘ and ‘REINIT’ commands.

## Overview

The COMM module translates between the standard interface described below and the audio mixer serial protocol. It parses the buffer for responses from the audio mixer, sends strings to control the audio mixer, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the command responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the Duet module.

TOA D2000

Duet

COMM Module

NetLinx

UI Module

Virtual

Device

SNAPI

Some functionality in the device interface may not be implemented in the API interface.  In cases where device functions are desired but not API-supported, the PASSTHRU command may be used to send any and all device-protocol commands to the device. See the PASSTHRU command and the Adding Functions to Modules section for more information.

A sample UI module and a touch panel file are provided in the module package.  These are not intended to cover every possible application, but can be expanded as needed by a dealer to meet the requirements of a particular installation.

## Implementation

To interface to the Alero_Comm_dr1_0_0.jar module, the programmer must perform the following steps:

- Define the device ID for the audio mixer that will be controlled.

- Define the virtual device ID that the COMM module will use to communicate with the main program and User Interface. Duet virtual devices use device numbers 41000 - 42000.

- If a touch panel interface is desired, a touch panel file Alero.TP4 and module Alero Main.axs have been created for testing.

- The Duet ClearOne_InteractPRO_Comm_dr1_0_0.jar module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module.

An example of how to do this is shown below.

DEFINE_DEVICE

dvAudioMixer = 5001:1:0

vdvAudioMixer1 = 41001:1:0

vdvAudioMixer2 = 41001:2:0

vdvAudioMixer3 = 41001:3:0

vdvAudioMixer4 = 41001:4:0

vdvAudioMixer5 = 41001:5:0

vdvAudioMixer6 = 41001:6:0

vdvAudioMixer7 = 41001:7:0

vdvAudioMixer8 = 41001:8:0

vdvAudioMixer9 = 41001:9:0

vdvAudioMixer10 = 41001:10:0

vdvAudioMixer11 = 41001:11:0

vdvAudioMixer12 = 41001:12:0

vdvAudioMixer13 = 41001:13:0

vdvAudioMixer14 = 41001:14:0

vdvAudioMixer15 = 41001:15:0

vdvAudioMixer16 = 41001:16:0

vdvAudioMixer17 = 41001:17:0

vdvAudioMixer18 = 41001:18:0

vdvAudioMixer19 = 41001:19:0

vdvAudioMixer20 = 41001:20:0

vdvAudioMixer21 = 41001:21:0

vdvAudioMixer22 = 41001:22:0

vdvAudioMixer23 = 41001:23:0

dvTP = 10001:13:0

DEFINE_START

DEFINE_MODULE 'GainComponent' mGainCmp1(vdvAudioMixerArray, dvTP, nGainBtns, nGainLevels, nGainInBtns, nGainLevelStore, nGainFBBtns, strGainInBtnsNames, nGainMuteNotSupportedBtns)

DEFINE_MODULE 'ModuleComponent' mMdlCmp1(vdvAudioMixer1, dvTP, nModuleButtons, nModuleVTButtons)

DEFINE_MODULE 'VolumeComponent' mVolCmp1(vdvAudioMixerArray, dvTP, nVolBtns, nVolLevels, nVolOutBtns, nVolPresetBtns, nVolLvlStore, nVolPsetStore, nVolFBBtns, strVolOutBtnsNames)

DEFINE_MODULE 'Alero_Comm_dr1_0_0' mAudMxrDev1(vdvAudioMixer1, dvAudioMixer)

Upon initialization, the AMX Comm module will communicate with the audio mixer and information will be exchanged. 
Port Mapping

This module uses multiple virtual devices in order distinguish events for one zone from another.

The ports on the virtual device are used to determine which Alero you wish to control and which Alero is reporting feedback. Since the functionality of each unit type may be different, the channels documented below may not be all applicable to the device you are controlling. If an unsupported command is sent, the command may be ignored or an error message will be reported depending on the type of command and the device.

| Virtual Device | Channels | Levels | Control | Feedback |
| --- | --- | --- | --- | --- |
| 41001:1:0 | All Channels | All Levels | All Control Cmds | All Feedback Cmds |
| 41001:<n>:0 Where n refers to the Gain/Volume components AudioMixer: n = 1: Input Assignment n = 2: Output Assignment Refer to the table below for the number of Components. | All except 145,146,201,202,204,205,208,238,239,240,251,252 | All Levels | NA | NA |

Table 1 - Port Mapping

| Inputs |  |
| --- | --- |
| Interface | Module Port |
| Mic 1 - Digital | 1 |
| Mic 2 - Digital | 2 |
| Mic 3 - Digital | 3 |
| Mic 4 - Digital | 4 |
| Mic 5 - Digital | 5 |
| Mic 6 - Digital | 6 |
| Mic 7- Digital | 7 |
| Mic 8- Digital | 8 |
| Line In 1 - Digital | 9 |
| Line In 3 - Digital | 10 |
| VC In - Digital | 11 |
| USB - In | 12 |
| Mic 1 - Analog | 13 |
| Mic 2 - Analog | 14 |
| Mic 3 - Analog | 15 |
| Mic 4 - Analog | 16 |
| Mic 5 - Analog | 17 |
| Mic 6 - Analog | 18 |
| Mic 7 - Analog | 19 |
| Mic 8 - Analog | 20 |
| Line In 1 - Analog | 21 |
| Line In 2 - Analog | 22 |
| VC In - Analog | 23 |

| Outputs |  |
| --- | --- |
| Interface | Module Port |
| Line Out 1 | 1 |
| Line Out 2 | 2 |
| USB - Out | 3 |
| VC Out | 4 |
| PA Out | 5 |

Table 2 –Input/Output Information

## Channels

The UI module controls the Alero via channel events (NetLinx commands pulse, on, and off) sent to the COMM module. The channels supported by the COMM module are listed below. These channels are associated with the virtual device(s) and are independent of the channels associated with the touch panel device.

Note: An ‘*’ indicates an extension to the standard API.

| Channel | Description |
| --- | --- |
| 24 | ON: Ramp Volume Up – used for feedback also OFF: Stop Ramping |
| 25 | ON: Ramp Volume Down – use for feedback also OFF: Stop Ramping |
| 26 | PULSE: Cycle Volume Mute |
| 140 | ON: start Gain ramping up OFF: stop Gain ramping Used for control and feedback. |
| 141 | ON: start Gain ramping down OFF: stop Gain ramping Used for control and feedback. |
| 143 | ON: Set Gain Mute ON OFF: set Gain Mute OFF Used for control and feedback. |
| 144 | PULSE: Cycle Gain mute state |
| 199 | ON: Set Volume Mute On – used for feedback also OFF: Set Volume Mute Off |
| 251 | ON: Device is Online – used for feedback only OFF: Device is not Online |
| 252 | ON: Data is Initialized – use for feedback only OFF: Data is not Initialized |

Table 2 - Virtual Device Channel Events

## Levels

The UI module controls the Alero via level events (NetLinx command send_level) sent to the COMM module. The levels supported by the COMM module are listed below. These levels are associated with the virtual device(s) and are independent of the levels associated with the touch panel device.

Note: An ‘*’ indicates an extension to the standard API.

| Level | Description |
| --- | --- |
| 1 | Volume Level (range 0…255) |
| 5 | Gain Level (range 0…255) |

Table 3 - Virtual Device Level Events

## Command Control

The UI module controls the Alero via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

Note: An ‘*’ indicates an extension to the standard API.

| Command | Description |
| --- | --- |
| ?DEBUG | Request the state of the debug feature. ?DEBUG |
| DEBUG-<value> | Set the state of debugging messages in the UI module and the Comm. module. Note: See Programming Notes section. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning & info messages on 4 = set all messages on DEBUG-1 |
| ?PROPERTY-<key> | Get the value of a property <key>. If the value is not initialized, an empty string is returned. <key> : IP_Address, Ramp-Analog-Step, Ramp-Digital-Step ?PROPERTY-IP_Address |
| PROPERTY-<key>,<value>> | Set the value of property <key> to <value>. This must be followed by the REINIT command to take effect. These values are not initialized by default. <key> : IP_Address, Ramp-Analog-Step, Ramp-Digital-Step <value> : string representing an IP address PROPERTY-IP_Address,10.0.0.5 |
| REINIT | Re-initializes the communication link and data. Note: This command deletes any messages waiting to go out to the device. REINIT |
| ?VERSION | Query for the current version number of the Duet module. ?VERSION |

Table 4 – Send Command Definitions

## Command Feedback

The COMM module provides feedback to the User Interface module for audio mixer changes via command events. The commands supported are listed below.

PLEASE NOTE: Feedback is only provided when there is a state change. If no state change resulted from the command sent in, then no feedback will be returned.

| Command | Description |
| --- | --- |
| DEBUG-<value> | Returns the state of debugging messages in the UI module and the Comm. module. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning and info messages on 4 = set all messages on DEBUG-1 |
| PROPERTY-<key>,<value> | Feedback on the value of property <key>. Note: An empty string is returned if the property has no value. <key> : IP_Address <value> : string representing an IP address <key> : Ramp-Analog-Step <value> : 1..44 – Step size for Gain/Volume Ramp. Default : 2.2 <key> : Ramp-Digital-Step <value> : 1..100 – Step size for Gain/Volume Ramp. Default : 3 PROPERTY- IP_Address,10.11.12.13 |
| VERSION-<version> | Reports the version number of the module. <version> : xx.yy.zz = module version number VERSION-1.0.0 |

Table 5 - Command Feedback Definitions

## Programming Notes

- PLEASE NOTE: In order to establish a communication connection between the module and the device, the module must know what IP address to connect to. Use the PROPERTY- command to set the module’s IP address and then use the REINIT command to force it to take effect. Here is an example of how these two commands are used:

Send_Command 41001:1:0, ‘PROPERTY-IP_Address,192.168.100.85’

Send_Command 41001:1:0, ‘REINIT’

Note that your virtual device (41001:1:0) and the IP address of your device (192.168.100.85) may differ from this example. Substitute the appropriate values where necessary.

## Protocol Notes

- The protocol offers asynchronous feedback whenever a change of state occurs.