ARCHITECTURAL SPECIFICATIONS

AMX Varia Touch Panel

VARIA-SL80 (AMX-UTP0801)

SUMMARY

- Professional-grade touch panel that features an ultra-slim design optimized for in-wall installations, utilizing standard single-gang electrical boxes. The touch panel is designed to adapt to the unique needs of your environment by offering a selection of personas. Varia personas are pre-loaded apps that define the entire panel experience.

PRODUCT IDs

- The touch panel shall be manufactured by AMX and model shall be VARIA-SL80.

FEATURES

- Persona-Defined Experience – Pre-loaded Web Kiosk, Zoom Rooms Controller, AMX Book Room Scheduling, and AMX G5 Control Personas transform Varia into the ideal customizable panel to serve the unique needs of any user

- Advanced security protocols - Includes the most advanced security protocols, including HTTPS, 802.1X, Modern Authentication or OAuth 2.0, and ability to support upcoming security standards TLS 1.3 & FIPS 140-3

- Integrated wizard software streamlines initial setup and ensures seamless future support through mass configuration and updates.

TOUCH SCREEN DISPLAY REQUIREMENTS

- Touch panel shall offer a resolution of 1280x800 with an aspect ratio of 16:10 landscape

- Touch panel display shall be a TFT Active Matrix Color LCD with In-Plane Switching Technology (IPS).  Touch panels not utilizing TFT, IPS displays, shall not be accepted.

- Touch panel display shall support a brightness of 350 nits (cd/m2). Displays with brightness levels lower shall not be accepted.

- Touch panel display brightness shall be adjusted either manually, or automatically through a photosensitive light detector in the panel. Touch panels not supporting such display brightness adjustment capabilities shall not be accepted.

- Touch panel display shall support a contrast ration of 800:1. Displays with lower contrast ratios shall not be accepted.

- Touch Overlay should offer Capacitive touch; >5 points touch panel with palm rejection and wet finger operation. Anti-fingerprint; Anti-microbial coating.

MEMORY REQUIREMENTS

- Touch panel shall have a minimum of 4GB, LPDDR4 and 16GB eMMC memory for project files and 3rd party apps. Anything lower shall not be accepted.

COMMUNICATION REQUIREMENTS

- Touch panel shall be equipped with an integrated Near Field Communications (NFC) read/write. Touch panels not equipped with a NFC reader, or requiring an external reader, shall not be accepted.

- Touch panel shall support Ethernet communications via a 10/100 port with RJ45 connector.

- Ethernet shall support dynamic and static IP addressing. Ethernet implementations that do not support both dynamic and static IP addressing shall not be accepted.

- Touch panel USB port connectivity shall allow for firmware upgrades, touch panel file transfer, JPEG image viewer, HID Peripherals, and USB audio output for headsets. Touch panels with USB ports not allowing for the aforementioned capabilities shall not be accepted.

VIDEO REQUIREMENTS

- Touch panel shall be equipped with a built-in High Definition 5MP front-facing with fixed focusing lens video camera. Touch panels not equipped with a built-in video camera, or requiring an external camera shall not be accepted.

- Touch panel shall be capable of simultaneously encoding and decoding (1) video stream. Touch panels not supporting at least (1) simultaneous encode and (1) decode stream shall not be accepted.

- Touch panel shall have configurable side LED’s programmable in any colour with level 0-100 for red, green and blue.

- Touch panel shall contain a proximity sensor and ambient light sensor

TECHNICAL SPECIFICATIONS

DIMENSIONS

- 8 13/32” x 5 1/32 x 1 1/16” (213.22mm x 127.64mm x 26.65mm)

WEIGHT

- 17 oz (482 g)

REGULATORY COMPLIANCE

- UL

- CE

- CB

- FCC Class B (-5dB)

INCLUDED ACCESSORIES

- Mounting Plate

- Screw Set

- Mounting Hardware

- Quick Start Guide

ACTIVE POWER REQUIREMENTS

- RJ-45, POE 802.3af, 15.4W

ENVIRONMENTAL

- Temperature (Operating)  0°C - 40° C (32°F – 104°F) • 5% - 85% Humidity

- Temperature (Storage) -20°C – 60° C (4°F – 140°F) • 5% - 90% Humidity