AMX Metreau 13-Button Ethernet Expansion Keypad
MET-13X-WH (FG5793-12-WH), White Keypad 
MET-13X-BL (FG5793-12-BL), Black Keypad 
ETHERNET KEYPAD REQUIREMENTS

SUMMARY
· Keypad shall be linkable to an Ethernet Keypad and combine multiple tactile control surfaces into an aesthetically-pleasing and logically-laid out interface. Keypad shall be designed to be approachable, allowing users to easily and intuitively access multiple system functions.
COMMUNICATION REQUIREMENTS

· Keypad shall be linkable to an Ethernet Keypad, and be independently addressable, without requiring an additional IP address. Keypads that are not linkable to an Ethernet keypad, or requiring an IP address to be independently addressable, shall not be accepted.
POWER REQUIREMENTS

· Keypad shall be powered by the Ethernet Keypad that it’s linked to. Keypads requiring an independent power supply shall not be accepted.

TACTILE SURFACE REQUIREMENTS
· Keypad shall be equipped with (13) push-buttons. Keypads with more or less than (13) push-buttons shall not be accepted.

· Keypad’s lowermost push-button shall be equipped with two lateral sensors, so as to enable presses to these sensors to be interpreted as discrete events. Keypads not equipped with at least (1) push-button with dual lateral sensors shall not be accepted.

· Keypad push-button silk screen labels shall be customizable with up to (18) alphanumeric characters. Keypads with fixed button configurations, or not supporting custom silk screen labels with at least (18) alphanumeric characters shall not be accepted.
FEEDBACK REQUIREMENTS

· Keypad shall be equipped with buttons capable of providing visual feedback via integrated LED lights. Keypads not capable of providing visual feedback via integrated LED lights shall not be accepted.
· Keypad visual feedback shall be distinctly programmable as on, off, or flashing. Keypads not capable of providing distinctly programmable visual feedback in the same manner shall not be accepted.
· Keypad pushbutton LED light brightness shall be adjustable so as to provide a proper level of backlighting, with a discernible on and off state. Keypads without adjustable feedback LED lights shall not be accepted.
MOUNTING REQUIREMENTS

· Keypad shall occupy a US standard, single-gang, electrical box space. Keypads requiring a custom back-box shall not be accepted.

PRODUCT IDs
· The Expansion Keypad shall be manufactured by AMX and shall be a Metreau MET-13X-WH (White).

· The Expansion Keypad shall be manufactured by AMX and shall be a Metreau MET-13X-BL (Black).

About AMX
AMX hardware and software solutions simplify the implementation, maintenance, and use of technology to create effective environments. With the increasing number of technologies and operating platforms at work and home, AMX solves the complexity of managing this technology with reliable, consistent and scalable systems. Our award-winning products span control and automation, system-wide switching and audio/video signal distribution, digital signage and technology management. They are implemented worldwide in conference rooms, homes, classrooms, network operation / command centers, hotels, entertainment venues, broadcast facilities, and more.  ©2014 AMX. All rights reserved.

Specifications subject to change.  17 June 2014.

AMX.com   |   800.222.0193   |   469.624.8000   |   +1.469.624.7400   |   fax 469.624.7153
